Assignment-5

16-bit MAC

Vagadia Yash Dinesh

Roll Number:20D070086

Course:EE671 VLSI Design

Q-1

Designing a 16-bit MAC with two 8-bit numbers as a input and a 16-bit number C as a input, in VHDL and simulating it using a test bench.

Dot diagram of MAC, wire reduction using DADDA scheme. From the 1st stage wires are named Colj(i), where j column number and i the index for element in that column.

From stage-2 output are named Wj(i) and same way from stage-3,4,5 as X,Z,Y. These are wires are defined as signals at the start of architecture.

Full adders, Half adder and logarithmic adder entities are defined in Gates.vhdl file using the given basic gates.

a.

An 8 cross 8 array of and gates is defined to generate partial product bits.

b.

Then Half adders and full adders are used for wire reduction as shown in dot diagram.

c. At finally 16-bit logarithmic adder is used from assignment-4 to final answer. From DUT final product and Carry is output.

There are 4 files in the project made in Quartus-MAC, DUT, Testbench and Gates

In MAC, structural code is written, DUT is for test the device.

Tracefile.txt (in which input combinations are written) is generated using python tracefile.py

I am using 20 random combinations of inputs for verification and a max value.

For an input—10101100000001111010001110011110 and output-01010100001010010

In simulation it starts at 247 ns and settles at 249.8 ns, so 2.8 ns delay to generate final output.

DUT code

Testbench code

Output file is generated at simulation/modelsim/output.txt

All the cases passed for the given random combination of input with no error.

RTL simulation